Specifications for 6 Stainless tanks of 15000 gallons

- Connection with valve for mobile agitator

  - Reference for agitator: https://boutique-bellot.com/melangeur/melangeur-agitateur-kiesel_p371-185#/64-type-rg3_o50_macon

- Closed dome with protected air intake against fire (End of line deflagration arrester)  Reference https://www.protectoseal.com/product/vapor-and-flame-solutions/flame-detonation-arresters/end-of-line-deflagration-flame-arresters/avc-series/

- Connection for filling pipe with a diameter of DN40

- Connection for washing ball installation centered at the upper dome

  - Reference: https://www.alfalaval.fr/produits/transfert-des-fluides/equipements-de-nettoyage-de-cuves/tetes-de-lavage-rotatives-par-impact/sani-sb/

- Maximum height: 5.5 meters including the legs

- Maximum diameter: 4 meters

- Capacity : 15000 gallons ± 10%

- Steel grade: 2 quotes, one for 316 L and another for 304 L

- Lifting ring if necessary to move the tanks with cranes

- Tasting valve in stainless steel

  - Reference: https://boutique-bellot.com/robinets-degustateurs/rob-degust-eco-inox-316-15x21-grand-debit-dem_p435

- Level jacks for leveling the tank feet

- Stainless rule +gauge+ valve to evaluate filling level

- Front ladder bar fixation on the top, near the gauge.

- Lateral manhole with watertight, food-grade seal

- Manhole on top of the tank, off-centered in front of the ladder bar fixation

- Interior finish type 2B or equivalent.

Process for assembly

- All metal parts must be made of stainless steel. The parts directly in contact with the liquid must be entirely in 316 L or 304 L according to the requested proforma.

- Quality certificates will be requested for stainless steel sheets, stainless equipment and welding electrodes before assembling the tanks.

- The contractor must specify the delivery date of the materials after acceptance of the offer. He will also mention which operations will be carried out directly on the site (assembling, welding, polishing, etc.), in how many days and what type of labor will be necessary on site.

- Drawing should be sent, approved and signed by Barbancourt before begin the construction.

- The price of transport must be estimated without taking into account customs fees.